X-Axis [3.0]

Bearings [3.1]

Parts Needed

1x    X-Axis (3D Printed)

2x    Standoffs (3D Printed)

10x  R4ZZ Bearing Washers (3D Printed)

2x    1/4” - 1.5” Bolts

4x    1/4” - 2” Bolts

4x    1/4” - 3/4” Set Screw

2x    1/4” Washers

2x    1/4” Lock Nuts

6x    R4ZZ Bearings

Use the longer bolts to thread into the ends of the X-Axis part. Short bolts are used in the adjustment slots.

PRO TIP: Use a 2.5” bolt instead of a 2” bolt and cut off the extra 1/2” to provide a smooth surface for the inner bearing race to ride on. Also use a 1.75” bolt instead of a 1.5” bolt and cut the extra 1/4” off.

Insert the 4 set screws, these are used to set the plumb of the X-axis

Motor Plate [3.2]

Parts Needed

1x    X-Axis with Bearings

2x    Standoffs (3D Printed)

2x    R4ZZ Bearing Washers

1x    X-Motor Plate

2x    1/4” - 1.5” Bolts

2x    1/4” Lock Nuts

3x    #10-32 - 1/2” Screws

Pro Tip: Grind bolts off in the green area so that they are flush to the motor plate if they are not flush already

X-axis must be plumb to the bed surface. Process:  Adjust the lower adjustable bearings so they are not touching the tube. Slightly tighten 1/4" bolts on upper adjustable bearings. Use a square to measure from bed to aluminum torch bar. Adjust the upper set screws to achieve plumb.  Firmly tighten 1/4" bolts. Adjust lower bearings so they just make contact with the tube and check that they spin when moving the assembly. Firmly tighten the lower 1/4" bolts on the adjustable bearings. Check that there is no play or rack on the X-axis bearing assembly.

Torch Holder [3.3]

Parts Needed

1x    X-Axis with Motor Plate

1x    Torch Clamp (3D Printed)

1x    Torch Holder (3D Printed)

1x    Torch Bar

1x    #10-32 - 3/4” Screw

2x    1/4” - 3/4” Bolt

1x    1/4” - 1/2” Bolt

2x    1/4” Washer