Motors and Belts [4.0]

X Motors and Belts [4.1]

Parts Needed

1x    X-Axis Assembly

1x    Small Stepper Motor

4x    #10-32 - 3/8” Screw

4x    608zz Bearing Washers (3D Printed)

2x    5/16” Nuts

4x    5/16” Fender Washers

4x    608ZZ Bearings

2x    5/16 - 1.5” Bolts

Y Motors and Belts [4.2]

Parts Needed

1x    Completed Gantry

1x    Large Stepper Motor

4x    608zz Bearing Washers (3D Printed)

4x    #10-32 - 5/8” Screws

4x    #10-32 - 3/8” Screws

4x    #10 Washers

4x    608zz Bearings

2x    5/16” - 1” Bolts

2x    5/16” Nuts

2x    5/16” Fender Washers

Holes for mounting the motor plate are not already in the bearing blocks. Use this reference to position to properly align the Y-Motor plate so you can drill and tap the mounting holes in the bearing blocks. Drill and tap for #10-32 screws.

Longer #10 Screws are used for mounting the Y-Motor Plate. Shorter screws are used for mounting the motor.

Pro Tip: There are 2  5/16” nuts on the backside of the motor plate that hold on the belt rollers.