3D Printed Part Post Processing

Print List

2x   Y-Axis Bearing Blocks

1x   Gantry Flange

1x   Gantry Flange - Motor

24x Bolt Spacers

2x   Belt Tensioners

2x   Belt Mount Idler (Optional)

2x   Belt Mount Active (Optional)

1x   X-Axis Bearing Mount

1x   Torch Holder

1x   Torch Clamp

4x   Standoff

16x End Caps

NOTE: If you damage the threads on a 3d printed part they can be repaired by using brass threaded inserts. The link is an example of what can be used.

https://www.amazon.com/Hilitchi-Threaded-Embedment-Assortment-Projects/dp/B07VFZWWXY/ref=sr_1_4?gclid=Cj0KCQiAo-yfBhD_ARIsANr56g5ZLVRn0cwER9rngUpGf611IzU_-QyWnGM6HJqBKKL3l2UlOdYB0gYaAuQYEALw_wcB&hvadid=580876202520&hvdev=c&hvlocphy=9020758&hvnetw=g&hvqmt=e&hvrand=330495531328933387&hvtargid=kwd-247897830&hydadcr=952_1014986903&keywords=threaded+inserts&qid=1677438294&s=industrial&sr=1-4

Y-Axis Bearing Blocks

1 - Drill out all 4 holes with a 17/64” drill bit

2 - Tap holes all 8 with ¼” - 20 Tap

3 - Drill out holes with a 5/16” or 8mm drill bit, make sure you can insert bearing pins relatively easily.

Gantry Flange and Gantry Flange - Motor

https://www.youtube.com/watch?v=lhejn2-bwxc&list=PLWACT6mAqAyVz0Aqc3zoBX9DPsVudicCc&index=3

1:30 - 5:00

1 - Drill out all 4 holes with 17/64” drill bit

2 - Drill out all 16 holes with 13/64” drill bit

X-Axis Bearing Mount

1 - Drill all three holes with 13/64” drill bit

2 - Drill and tap all 10 holes with 1/4” - 20 tap

3 - Make sure a ¼” bolt can move freely in 4 these holes

4 - Drill and tap both holes with 5/16 -18 tap

PRO TIP: On step number 4 attach the motor plate first and tap both at the same time so the threads align

Belt Mount Idler and Belt Mount Active

1 - Drill hole with 5/16” or 8mm drill bit

2 - Drill out holes with 13/64” drill bit and make sure a #10 screw move freely in slots

Bolt Spacer

1 - Drill out hole with ⅜” drill bit

Belt Tensioner

1 - Drill and tap hole with ¼”-20 Tap

2 - Drill out both holes with 13/64” drill bit

Torch Holder

1 - Drill out both holes with 17/64” drill bit

Torch Clamp

1 - Drill and tap with #10-32 tap

2 - Drill and tap with 1/4” - 20 tap

Standoff

1 - Drill out hole with ¼” drill bit

End Cap

No post processing required